Document Version: 2001.001

Dear SPRINT user,

when you get the screen message "Please ask your distributor for MODxxx..., a hardware change is nescessary for that special device you want to program. The required change is easy to do, so we think you can do it by yourself. If you have any problems don't hesitate to call SMS or send us the TOP.

This modification is concerning some microcontrollers who have the clock- and supply-pins not at the standard locations. Look at the schematic on the page 2 of this document to locate the positions of the pins and the requested solder pads.

The TOP1PLC has two clock-sinals. The first is on pin 41 (of the 84-pin programming socket).
This is the mostly required location (51-family). The second one is pin 83 that is required for several
controllers (196-family). This signal is prepared to be rewired.
	- take off the TOP1PLC and screw out the four screws at the inside of the TOP.

- take out the PCB's and disconnect them.

clock-pin:	- cut the copper track in the 'X' field and solder a wire from the pad 'L1' to the required pin.

GND-pin:	Look for the pad 'G1', 'G2' and solder a wire from that pad to the required pin.

VCC-pin:	Look for the pad 'V1', 'V2' and solder a wire from that pad to the required pin.

WARNING : It is not posible to have two different modifications in the same TOP, that are using the same signals (e.g. V1, G1 etc). Only modifications that are using different signals can be installed in one TOP1PLC.  Examples : MOD8/92 and MOD2/93 in one TOP are not possible. MOD4/93 and MOD6/93 in one TOP are possible !

Attach the two PCB's together carefully. Put them back into the TOP and screw in the four screws. Start the PROM software and select the device to check your work. The message "Please ask your distributor...." should no longer appear on the screen.

NOTE : Gx (Vx) means --> select G1 or G2 (V1 or V2)

| MOD | Device | Signal | Pin-number |
| --- | --- | --- | --- |
| 1/92 | Philips 87C552 | CLOCK GND Gx VCC Vx | 43 44 2 |
| 2/92 | Philips 87C592 | CLOCK VCC Vx | 42 2 |
| 3/92 | Philips 87C451 | CLOCK GND Gx | 65 66 |
| 4/92 | several | CLOCK | original |
| 6/92 | several | CLOCK VCC Vx | 84 42 |
| 7/92 | several | CLOCK | 4 |
| 8/92 | 87C51GB | CLOCK | 64 |
| 2/93 | Intel 87C198 | CLOCK | 2 |
| 4/93 | Hit. 647532/4 | VCC Vx | 55 |
| 6/93 | Xilinx 7236 | GND | 17 |
| 7/93 | Philips 87C550 | CLOCK GND Gx | 43 44 |
| 9/93 | MC68HC705X16 | VCC Vx GND Gx | 38 2 |
| 10/93 | MC68HC705B16 MC68HC705B32 | VCC Vx GND Gx | 18 65 |

| 11/93 | Intel 87C196KT, Intel 87C196KQ Intel 87C196KR | VCC Vx GND Gx CLOCK | 39 5 original |
| --- | --- | --- | --- |
| 1/94 | SAB83C517AH-5J | CLOCK | 39 |
| 5/94 | Intel 87C196MC, Intel 87C196MD | VCC Vx CLOCK | 39 82 |
| 6/94 | Intel 87C196CA | CLOCK | 84 |
| 7/94 | Texas Instr. TMS320E25 | VCC Vx GND Gx | 77 14 |
| 8/94 | MC68HC711L6FN | VCC Vx | 42 |
| 9/94 | Intel 87C196NT | GND Gx CLOCK | 5 83 |
| 11/94 | MC68HC711K4 | VCC Vx | 2 |
| 12/94 | Intel 87C196KL/CB | CLOCK | 81 |
| 17/94 | Xilinx XC7336/54 -44 XC9536-44 | VCC Vx | 62 |
| 18/94 | XILINX XC7354 - 68 | VCC Vx GND Gx | 38 18 |
| 4/95 | Intel 87M12 SGS CC460 | VCC Vx GND Gx | 63 21 |
| 5/95 | Siemens SAB 83C515AH | GND Gx CLOCK | 46 47 |
| 11/95 | Xilinx XC7372-68 | VCC Vx | 38 |
| 12/95 | Xilinx XC7372-84 XC73108-84 XC95xxx-84 | VCC Vx GND Gx | 38 16 |
| 14/95 | TI TMS77C82 | VCC Vx | 48 |
| 21/95 | INTEL 8796JC | VCC Vx GND Gx CLOCK | 17 44 83 |
| 2/96 | INTEL 87C196KN | VCC Vx GND Gx CLOCK | 42 6 4 |
| 4/96 | National COP8780/8788/888 | VCC Vx GND Gx | 18 67 |
| 10/96 | Texas Instr. MSP430E313 | VCC Vx GND Gx | 16 7 |
| 11/96 | Texas Instr. MSP430E325/ MSP430P325FN | VCC Vx VCC Vx GND Gx | 14 9 7 |
| 16/96 | National COP87L88G/E/C | VCC Vx GND Gx | 18 67 |
| 17/96 | National COP87L88RW | VCC Vx VCC Vx GND Gx GND Gx | 15 51 16 56 |
| 22/96 | Hitachi HD647329 | VCC Vx VCC Vx | 40 50 |
| 7/97 | Microchip PIC 16C9xx | VCC Vx GND Gx | 80 7 |

| 8/97 | National COP87L88EB | VCC Vx GND Gx | 63 21 |
| --- | --- | --- | --- |
| 9/97 | National COP87L89EB | VCC Vx GND Gx | 62 21 |
| 21/97 | Dallas DS 87C530 | CLOCK | 40 |
| 2/98 | Microchip PIC17C756+17C752 | CLOCK | 62 |

Pin- and Solderpad Positions

Component side view of the upper PCB