Document Version: 2003.002

Dear SPRINT PLUS48 user,

when you get the screen message "Please ask your distributor for MODxx/yy " a hardware change is necessary for that special device you want to program. This modification is concerning some devices who have the supply-pins or fast clocks not at the standard locations. Look at the schematic below to locate the position of the pins and the "spare-clocks/relais" Sx on the PLUS48 PCB.

1.)	Disconnect the power- and the printerport cable from the PLUS48.

2.)	Remove the four screws on the sides of the PLUS48 case.

3.)	Remove the bottom sheet and insert an isolated wire from the required pin 
	(see following table) to the solderpoints Sx.

4.)	Attach the PLUS48, connect both cables and start the Sprint software. The screen 
	message should no longer appear, if the jumpers are in the right positions.

NOTES :	For the Fast Pullup, you can use one of the PADS S1, S2, S3 or S4.

The PAD S5 is a spare GND-Relay, PADs S6, S7 are spare VCC-Relay.

For the Oscilator (CLOCK), please remove JP10 (or JP11);

The centre-pin is the oscilator.

PAD S7 Can be configured as GND relay with Jumper setting JP9 open and JP8 closed, S7 is configured as VCC relay with JP9 closed and JP8 open.

WARNING : It is not posible to have two different modifications, at the same time, installed in the PLUS48, that are using the same signals (e.g. S1, S2, etc.). Only modifications that are using different signals can be installed, at the same time, in the PLUS48.  Examples : MOD 09/95 and MOD15/95 in the PLUS48 are not possible. MOD09/95 and MOD14/94 in the PLUS48 are possible !

PLUS48 PCB solderpoints positions

Pin Connection Table

| MOD # | Device | Solderpoint | Pin # |
| --- | --- | --- | --- |
| 1 | several | set JP2 |  |
| 2 | several | set JP6 |  |
| 3 | several | set JP3 and JP7 |  |
| 4 | several | set JP10 |  |
| 14/94 | Intel 28F020, AMD 29F040 | S2 | 39 |
| 09/95 | PIC 16C74 | S6 | 16 |
| 15/95 | Z86E02/04/07/08 | S6 | 20 |
| 16/95 | Altera EPM7064-44 | S1 or S2 or S3 or S4 | 3 |
| 18/95 | Cypress PALCE16V8 | S1 S2 | 19 20 |
| 20/95 | AM27S27 | S6 | 35 |
| 8/96 | Philips ABT22V10A LVT22V10 DIP | solder 10K resistor between S6 and pin 33 between S7 and pin 34 JP9 closed JP8 open |  |
| 9/96 with S491 | Philips ABT22V10A LVT22V10 PLCC | solder 10K resistor between S6 and pin 40 between S7 and pin 31 JP9 closed JP8 open |  |
| 12/96 | COP87L22/42 | S6 | 20 |
| 13/96 | COP87L20/40/84 | S6 | 16 |
| 14/96 | COP87L88, Flash devices with ET48-48-03TS | S5 | 37 |
| 15/96 with S493 | Philips PZx032/PZx064 | S6 S5 | 23 39 |
| 18/96 | SGS M92256-14 | S5 | 23 |
| 21/96 | MC68HC705J1A | S6 | 23 |
| 1/97 | Xilinx XC9536/72 | S6 S7 | 20 22 |
| 11/97 with S491 | Atmel ATF22V10B/C | S1 or S2 or S3 or S4 | 24 |
| 13/97 not with new ASICs | Atmel ATF1500/A | S1 or S2 or S3 or S4 | 6 |
| 19/97 | Altera EPCxxxx | S5 | 25 |
| 23/97 | Intel 87C42 (DIL) | CLOCK | 7 |
| 24/97 | Intel 87C42 (PLCC with S493) | CLOCK | 13 |
| 17/98 | Xilinx XC17256L | S5 | 25 |
| 18/98 | MC68HC908QLxP | CLOCK | 30 |
| 7/99 | AT45DB011/D011-SOIC(8pin) | S5 S6 | 27 26 |
| 8/99 | AT45DB081/D081-TSOP(32pin) | S6 S5 | 39 40 |
| 10/99 | AS29F080 –SO(44 pin) | S6 S5 | 3 11 |
| 11/99 | LH28F400BVE-TSOP(48pin) ET-Adapter: AS48-48-03T | S7 and JP8 closed(GND) and JP9 Open! S5(GND) | 37 25 |
| 12/99 | TI MSP430P112 | S6 | 16 |
| 13/99 | ST72xxx | CLOCK S5 (GND) | 12 37 |
| 05/2000 | MC68HC908JK1 /JK3 | S5 (GND) CLOCK | 16 17 |
| 2/2001 | SST 45xVxx | S5 (GND) S6 (VCC) | 27 22 |
| 5/2002 | MC68HC908QY | CLOCK | 20 |
| 5/2003 | # AT45DBxxx SOIC-8 | S5 (GND) S6 (VCC) | 27 26 |
| 6/2003 | MC68HC908QTxP | CLOCK | 26 |