Subject:	Motorola MC9S12 microcontroller programming with SPRINT

Date:		12/12/2001

Author:	SK

Pages:	2

### MC9S12 (STAR12) microcontroller:

### Flash Programming issues

Since STAR12 microcontroller can have up to 256k flash memory, but the native address range is restricted to 64k, the modules will be mapped through an address window of 16 kbyte size. The additional address bits are located in the PPAGE register.

Since the allowed PPAGE register values are $30..$3F (currently) and S-records are physical addresses, the corresponding linear address space is $C0000-$FFFFF.

SPRINT programmer use this linear address space to address the up to 256kbyte Flash of the MC9S12 microcontroller.

There are up to 4 blocks of 64k bytes numbered from 0 to 3:

0 = F0000-FFFFF,	1 = E0000-EFFFF,	2 = D0000-DFFFF,	3 = C0000-CFFFF.

For mapping details please refer to Table 2.

##### Flash Protection/Option Fields

Flash block 0 also holds a field of 16-bytes containing Security, Protection and "backdoor" comparison bytes.

Table 1: Flash 0 Protection/Security Field

| Programmer Address | MCU Address | Description |
| --- | --- | --- |
| $FFF00-$FFF07 | $FF00-$FF07 | Backdoor comparison key |
| $FFF08-$FFF09 | $FF08-$FF09 | Reserved (set to FF) |
| $FFF0A | $FF0A | ###### Protection byte for Flash block 3 |
| $FFF0B | $FF0B | Protection byte for Flash block 2 |
| $FFF0C | $FF0C | Protection byte for Flash block 1 |
| $FFF0D | $FF0D | Protection byte for Flash block 0 |
| $FFF0E | $FF0E | Reserved (set to FF) |
| $FFF0F | $FF0F | Security Byte SEC[7:0] |

Security Byte:

The bits 1 and 0 of this byte defines the secure state of the microcontroller. Only the combination SEC[1:0] = 10 define the UNSECURED state, all other will prevent any access to the MC9S12 memory.

CAUTION ! Several mask sets (0K36N) can not be erased by the programmer, once
                   SECURED. Set the security state carefully. The error “ramload operation
                   failed” can also mean, you have a microcontroller in the SECURED state.

Table 2: Memory Map for MC9S12 C

|  |  |  | Programmer Address Range | MCU Address Range |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 00000 00800 00C00 00FFF | 00000 00800 00C00 00FFF | - ¦ ¦ - | - ¦ ¦ ¦ - | - ¦ ¦ ¦ ¦ - |  |  |  | ## EEPROM |
|  | # PPAGE | # BLOCK |  |  |  |  |  |  |  |  | ------------------------------------------------------- FLASH ----------------------------------------------------------> |
| 16k: | $30 | 3 | C0000 C3FFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $31 | 3 | C4000 C7FFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $32 | 3 | C8000 CBFFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $33 | 3 | CC000 CFFFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $34 | 2 | D0000 D3FFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $35 | 2 | D4000 D7FFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $36 | 2 | D8000 DBFFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $37 | 2 | DC000 DFFFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $38 | 1 | E0000 E3FFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $39 | 1 | E4000 E7FFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $3A | 1 | E8000 EBFFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $3B | 1 | EC000 EFFFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $3C | 0 | F0000 F3FFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $3D | 0 | F4000 F7FFF | 8000 BFFF |  |  |  |  |  |  |  |
| 16k: | $3E | 0 | F8000 FBFFF | 4000 7FFF |  |  |  |  |  |  |  |
| 16k: | $3F | 0 | FC000 FFFFF | C000 FFFF |  |  |  |  |  |  |  |