Document Version: 2005.002

Dear SPRINT user,

when you get the screen message "Please ask your distributor for MODxxx..., a hardware change is necessary for that special device you want to program. The required change is easy to do, so we think you can do it by yourself. If you have any problems don't hesitate to call SMS or send us the TOP.

This modification is concerning some devices that needs a fast pull-up on a special pin or that have the supply-pins not at the standard locations. Look to the schematic on the page 2 of this document to locate the positions of the pins and the requested solder pads.

1)	Remove the TOP48DIP of your programmer.

2.)	Connect the requested solder pad to the requested pin by an isolated wire

Note :	The solder pads are splitt into two different parts, one circle and one square - please

use always the square part to connect the isolated wire with the requested pin.

3.)	Connect the TOP to the programmer and reselect the device

The message "Please ask your distributor...." should no longer appears on the screen.

| MOD | Device | Signal | Pin-number |
| --- | --- | --- | --- |
| 15/94 | MC68HC711D3 | GND : JP5 | 5 |
| 16/94 | Z86E02/04/07/08 | VCC: JP7 | 20 |
| 20/94 | MC68HC11A1/A8 | GND: JP5 | 23 |
| 21/94 | SDA 2506 | VCC : JP7 | 23 |
| 13/95 | Cypress PALCE16V8 | S7 S8 | 19 20 |
| 20/95 | AMD Am27S27 | VCC: JP7 | 35 |
| 1/96 | SGS M92256 | GND: JP5 | 23 |
| 5/96 | Philips ABT22V10A LVT22V10 | solder a 10KW resistor between JP7 and pin 33 and between JP8 and pin 34 |  |
| 12/96 | COP87L22/42 | VCC : JP7 | 20 |
| 13/96 | COP87L20/40/84 COP8SAx7-28p | VCC: JP7 | 16 |
| 14/96 | COP87L88, Flash devices with ET48-48-03TS | GND: JP5 | 37 |
| 19/96 | PZx032 | GND: JP5 | 32 |
| 20/96 | MC68HC705J1A | VCC: JP7 | 23 |
| 4/97 | PZx064 | GND: JP5 | 44 |

| 13/97 not with new ASICs | Atmel ATF1500/A | S7 or S8 | 6 |
| --- | --- | --- | --- |
| 14/97 | COP SAx7-16 | VCC : JP7 GND: JP5 | 22 27 |
| 16/97 | Xicor 88C75 | VCC: JP7 | 48 |
| 22/97 | Intel 87C42 (DIL) | CLOCK: JP2 | 7 |
| 18/98 | MC68HC805PV8 MC68HC908QLx | CLOCK: JP2 | 30 |
| 19/98 | Motorola 68HC705CJ4 | JP1 or JP2 or JP3 or JP4 | 43 |
| 2/99 | Samsung KM29N040T | VCC : JP7 GND: JP5 | 44 5 |
| 4/99 | Atmel AT45dxxSOIC 28pin | GND: JP5 | 11 |
| 5/99 | Atmel AT45dxxTSOP 28 pin | VCC: JP7 GND: JP5 | 37 38 |
| 6/99 | Mitsubishi M37510EFFP 176 pin | VCC: JP7 | 39 |
| 7/99 | Xilinx 1704LVC | GND: JP5 GND: JP6 | 4 22 |
| 8/99 | AT45DB081/D081-TSOP (32pin) | VCC: JP7 GND: JP5 | 39 40 |
| 9/99 | M29W102BB (TSOP-40 pin) | VCC: JP7 GND: JP5 | 24 42 |
| 10/99 | AS29F080 (SO-44 pin) | VCC : JP7 GND : JP5 | 3 11 |
| 12/99 | TI MSP430P112 | VCC: JP7 | 16 |
| 13/99 | ST72T101xx | GND: JP5 CLK: JP2 | 37 12 |
| 5/2000 | MC68HC908JK1 /JK3 | GND: JP5 CLK: JP2 | 16 17 |
| 1/2001 | MC68HC908GP32 | GND: JP5 VCC: JP7 CLK: JP2 | 23 24 9 |
| 2/2001 | Atmel 89C51CC02 QFP 32 | GND:JP5 VCC:JP7 | 23 22 |
| 3/2001 | MC68HC908KX8 | GND: JP5 CLK: JP2 | 17 26 |
| 4/2001 | MC68HC908GR8 | CLK: JP2 GND:JP5 VCC: JP7 | 27 36 37 |
| 1/2002 | PIC17C4x | CLK: JP2 | 31 |
| 2/2002 | MC68701U4 | GND: JP5 VCC: JP7 CLK: JP2 | 5 11 7 |
| 4/2002 | NEC 78F0114 | GND: JP5 VCC: JP7 CLK: JP2 | 7 6 15 |
| 5/2002 | Motorola MC68HC908QY | CLK: JP2 | 20 |
| 6/2002 | CY8C26643 | VCC: JP7 | 15 |
| 1/2003 | STM ST72Flite0x | GND: JP5 CLK: JP2 | 17 24 |
| 2/2003 | NEC 78F9026 | CLK: JP2 | 7 |
| 3/2003 |  |  |  |
| 4/2003 | STM ST72F26x | GND: JP5 CLK: JP2 | 37 12 |
| 5/2003 | AT45DBxxx SO-8 | VCC: JP7 GND: JP5 | 26 27 |
| 6/2003 | MC68HC908QTx P | CLK: JP2 | 26 |
| 7/2003 | AT89C51CC02 SO-28 | VCC: JP7 AGND: JP5 | 13 26 |
| 8/2003 | AT89C51CC01 VQFP-44 | CLK: JP2 | 44 |
| 1/2004 | STM ST7Flite1x & St7FLite2x | VCC: JP7 CLK: JP2 | 16 17 |
| 2/2004 | STM ST7FLCD1 SO-28 | GND: JP5 VCC: JP7 CLK: JP2 | 23 24 26 |
| 4/2004 | CY22381 SO-8 | GND: JP5 VCC: JP7 | 26 23 |
| 5/2004 | CY2239x TSSOP-16 | GND: JP5 GND: JP6 | 19 27 |
| 6/2004 | Zilog ZLP32300H4832 SSOP-48 | GND: JP5 VCC: JP7 | 13 37 |
| 1/2005 | STM ST72F623F2M SO-20 | VCC: JP7 CLOCK: JP2 | 22 25 |
| 2/2005 | MC9S08RxXX DIP28/SO28 | #### CLK: JP2 | 32 |