#### DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

FAX: 425.867.6972

pmhelp@dataio.com

### Subject: SMAC HEAD GROUNDING

Model Affected:PP100

1.0	PURPOSE: To inform service personnel how to ground the SMAC amp cable.

2.0	TOOLS REQUIRED:  Soldering iron, knife, sandpaper, Allen wrench set.

3.0	PARTS REQUIRED: Braided ground wire, solder, wire lug.

- INSTRUCTIONS:  Cut away some of the plastic cover on the SMAC amp cable on the amp side.

Solder the braided ground cable to the shielding of the cable.

Attach a wire lug to braided cable on the end that is not soldered to the cable.

Attach the lug under the SMAC amp.

Remove the screw from the plate that the SMAC amps are on and with sandpaper remove the paint under the plate so that you will have a better ground.

Remove paint from here.

Put braided cable here

Put lug end of  braided ground cable here.