#### DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

FAX: 425.867.6972

pmhelp@dataio.com

### Subject: PP100 Checksum Error

- Purpose: To inform service personnel about possible error while loading large data files on the PP100. This can occur if the data file, on the I:\PRGDATA\ directory is large (for example more than 20MB).

## The error messages on AH400 could be:-

“Checksum does not match on programmer Number” or

“Error Initializing Programmer Number”

2.0	Tools Required: None

3.0	Parts Required: None

- Instructions: In order to rectify this error, the Programmer Initialization Time Out value has to be increased from the default of 200 to 300 or more.

  - Exit completely from AH400

  - On the Handler Computer directory C:\AH400_32\ open the file WinAH400.INI and after the line TotalProgrammers=Nbr add the following line:-

# PgmrInitTimeOut=200

and increase this to 250, 300, 350 or more - the exact value would depend on the size of the data file, number of programmers and the device selected.

  - After making the changes to the WinAH400.INI file, save it and restart AH400 and re-load the task.

Number: any value between 1 to 12, depending on the number of programmers installed.