DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98073

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

FAX: 425.867.6972

support@dataio.com

### Subject: PS300 Upgrade to TLWin V5.01 and AH400 V5.0.0

This document provides guidelines on upgrading PS300FlashCore and PP100 machines with FlashCore programmers to TaskLink Version 5.01 and AH400 Version 5.0

- Requirements:

- TaskLink CD – this CD has both

- TaskLink Ver. 5.01 and FlashCore FirmWare Ver. 05.00.00.C

- AH400 CD with AH400 Software Ver. 5.0.0

Procedure / Steps: -

- On Handler Computer backup AH400 INI and all Reference Vision and Package files, that is:-

C:\AH400_32\WinAh400.ini

C:\Ah400_32\Package\<all_files).txt

C:\AcuWin95\Test\<all_files>.prj

- If existing firmware on FlashCore programmers is 4.0.03.C or earlier:-

On Handler computer, press the left [Shift] key and insert the CD, while keeping the left [Shift] key depressed.

Explore the CD and from TaskLink CD extract these two files

D:\PSSeries\PS_Sys\SysFlash.bin

D:\PSSeries\PS_Sys\Boot.bin

Copy these two files to Handler Computer’s directory

C:\Dataio\Tlwin\PSSeries\PS_Sys\

Run the existing TaskLink software and from top menu go to Tools and “Update Programmer Firmware”

Once all programmers are updated with FirmWare 5.00.00.C, exit TaskLink and remove TaskLink CD.

- Install TaskLink on Handler Computer

Insert TaskLink CD into Handler Computer

This will autorun the Setup and installs new TaskLink on handler computer. Keep all default values while installing TaskLink and select “Yes, I want to restart my computer now” and then Finish. When Handler Computer re-boots, remove TaskLink CD.

- Install AH400 on Handler Computer

Insert AH400 CD into handler Computer.

Navigate to CD-ROM Drive, D:\Handler Computer\AH400_Install

Double click on Setup.exe to install AH400

Within Install Shield /Welcome box select Repair option and click on [Next]

If message “ReadOnlyFileDetected” appears, select [Yes]

Click [OK] and [Yes] to restart computer.

- Update / Edit WinAh400.ini file.

Using Notepad open C:\AH400_32\WinAH400.ini file and the backed-up version of this file.

Edit the WinAH400.ini so that it has all the options / values from the backed-up WinAH400.ini file. For example, if the system had 4 Programmers, then within WinAH400.ini the entry would be RRFlashPgmr=1,2,3,4

Similarly, if system had Tape-Out installed then the required entry should be

TapeOutputInstalled=TRUE

- Confirm if all reference vision files and package files are present. If not, copy them from the backed up version into their respective directories.