#### DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

FAX: 425.867.6972

pmhelp@dataio.com

### Subject: PP100 Programmer Lockup

Model Affected:PP100

1.0	PURPOSE: To inform Customers about a situation that can lock up the PC104 on the PP100.

2.0	TOOLS REQUIRED: NONE

3.0	PARTS REQUIRED:NONE

- Symptom:  1 or more programmers always fails to initialize at job start up.

- CAUSE:  A job was started with the wrong adapter installed and when you go to the Programmer you see the error message invalid device code.

- SOLUTION:  Remove the adapter and install the correct adapter.

  - Then exit AH400.

  - Then cycle the circuit breaker for the Programmers.

  - Make sure that the programmer is at the Sprint start up menu.

  - Then go to the M:\Prog1 or which ever programmer has the error and delete the file default.job.

  - Restart AH400 and the job will now run correctly.

menu.